| PHASE1 |  |  |  |
| --- | --- | --- | --- |
| Project name |  |  |  |
| Document ref |  |  |  |
| Version |  |  |  |
| Release date |  |  |  |
| Author | Wu Fei |  |  |
| Classification |  |  |  |
| Distribution List |  |  |  |
| Approved by | Name | Signature | Date |

Ming Hsieh Department of Electrical Engineering

University of Southern California, Los Angeles, CA 90089

Spring 2022

EE 477

Revision History

| Date | Version | List of changes | Author + Signature |
| --- | --- | --- | --- |

C A T A L O G

1 Motivation	1

2 Part 1: Multiplier	2

# Motivation

Motivation

Multipliers and dividers are common arithmetic operations performed using the basic units adder and subtractor. For the final project, you will design a multiplier and divider to operate on 4-bit inputs.

# Part 1: Multiplier

Notes:

Input A = {a3a2a1a0}, Input B = {b3b2b1b0}, Output product Z = {z7z6z5z4z3z2z1z0}

FA = Single bit Full Adder,

HA = Single bit Half Adder

All blue-colored logic gates are 2-input AND

Procedure:

Design the schematic for the 4x4 multiplier as shown.

Do functionality verification using at least 3 input combinations.

Design a delay measurement circuit where each input has inv_1X preceding it.

Connect inv_4X to output z5. Use this input transition:

b3,b2,b1,b0,a3,a2,a1 = 1 1 1 1 1 0 1 (fixed)

a0 goes from 0->1->0

This will make z5 go from 0->1->0

Measure the rising and falling propagation delay from a0 to z5

Remember: Given a and b values are inputs to the actual multiplier, so inputs to the preceding inverters should be the opposite.